# CPE403 – Advanced Embedded Systems

# Design Assignment 01

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

Name: Rishawn Peppers Johnson

Email: Peppersj@unlv.nevada.edu

Github Repository link (root): https://github.com/PeppersJ/v4e0nk_i3

Youtube Playlist link (root): https://drive.google.com/drive/folders/1fJ029-AAWjTnN-QrRqNLd0iLwKGm6A08?usp=sharing

Follow the submission guideline to be awarded points for this Assignment.

Submit the following for all Assignments:

- In the document, for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only.

- Create a private Github repository with a random name (no CPE/403, Lastname, Firstname). Place all labs under the root folder TIVAC, sub-folder named Assignment1, with one document and one video link file for each lab, place modified c files named as asng_taskxx.c.

- If multiple c files or other libraries are used, create a folder asng1_t01 and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) with startup_ccs.c and other include files, c) text file with youtube video links (see template).

- Submit the doc file in canvas before the due date. The root folder of the github assignment directory should have the documentation and the text file with youtube video links.

- Organize your youtube videos as playlist under the name “cpe403”. The playlist should have the video sequence arranged as submission or due dates.

- Only submit pdf documents. Do not forget to upload this document in the github repository and in the canvas submission portal.

- Code for Tasks. for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only. Use separate page for each task.

Task 1

/*  Created By: Rishawn Peppers Johnson

*  Date Created: 01 October 2020

*  Device: TivaC123GH6PM

*  CpE 403 Assignment 01 Task 01

*

*  Purpose: Constantly display temperature through the terminal using a timer interrupt every 0.5s.

*          The PF4 button will toggle all three (R,G,B) LEDs using an interrupt.

*

*  Inputs: None

*

*  Outputs: Current temperature in F and C to terminal through UART.

*

*  */#include <stdint.h>     // Variable definitions for C99 Standard

#include <stdbool.h>    // Boolean definitions for the C99 Standard

//#define TARGET_IS_BLIZZARD_RB1    // For rom.h, defines TivaC type

#include "inc/tm4c123gh6pm.h"   // def. for the interrupt and register assignments on the Tiva C Series device on the launchPad board

#include "inc/hw_memmap.h"      // Macros defining the memory map

#include "inc/hw_types.h"       // Common macros for TivaC

//#include "inc/hw_ints.h"      // Macros for TivaC interrupts

//#include "driverlib/rom.h"    // Macros for calling functions in ROM

#include "driverlib/rom_map.h"  // Macros for default calling functions in ROM otherwise in Flash

#include "utils/uartstdio.h"    // UART Console Driver Functions

#include "driverlib/interrupt.h"// Macros for NVIC Controller API of DriverLib

#include "driverlib/gpio.h"     // API GPIO ports

#include "driverlib/debug.h"    // Macros for debugging driverlib

#include "driverlib/sysctl.h"   // Driver for SysTick

#include "driverlib/systick.h"  // Driver for SysTick timer in NVIC

#include "driverlib/adc.h"      // Drivers for ADC

#include "driverlib/timer.h"    // Drivers for timers

#include "driverlib/uart.h"     // Macros for UART

#include "driverlib/pin_map.h"  // Mapping of peripherals and pins

// Library error routine

#ifdef DEBUG

void __error__(char *pcFilename, uint32_t ui32Line) {

}

#endif

// Pins

#define RED_LED   GPIO_PIN_1

#define BLUE_LED  GPIO_PIN_2

#define GREEN_LED GPIO_PIN_3

// Variables

uint32_t ui32Period;

uint32_t ui32ADC0Value[4]; // Stores ADC value (size of ADC sequencer)

volatile uint32_t ui32TempAvg;

volatile uint32_t ui32TempValueC;

volatile uint32_t ui32TempValueF;

// Prototypes

// Initialize Functions

void init_ADC();

void init_GPIO();

void init_TIMER();

void init_UART();

// Interrupt Handlers

void Timer0AIntHandler(void);

void GPIOF0IntHandler(void);

// Program Entry

int main (void) {

//System clock to 40Mhz (PLL= 400Mhz / 10 = 40Mhz)

MAP_SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

// Enable Peripherals

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0); // Running at default rate of 1Msps

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER0);

// Configure Peripherals

init_ADC();

init_GPIO();

init_TIMER();

init_UART();

UARTprintf("Terminal Active\n");

IntMasterEnable();                  // Enable Master Interrupt

TimerEnable(TIMER0_BASE, TIMER_A);  // Enable Timer 0

ADCSequenceEnable(ADC0_BASE, 1);    // Enable Sequencer 1

while(1) {

}

}

void init_ADC() {

ADCSequenceConfigure(ADC0_BASE, 1, ADC_TRIGGER_PROCESSOR, 0); // Using ADC0, sequencer 1, processor triggered, highest priority

// All 4 sequencers sampling internal temperature sensor

ADCSequenceStepConfigure(ADC0_BASE, 1, 0, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 1, 1, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 1, 2, ADC_CTL_TS);

// Samples temperature, sets interrupt flag when done, tell ADC last conversion on sequencer 1

ADCSequenceStepConfigure(ADC0_BASE, 1, 3, ADC_CTL_TS|ADC_CTL_IE|ADC_CTL_END);

}

void init_GPIO() {

// Initialize GPIO pins

GPIO_PORTF_LOCK_R = 0x4C4F434B;   // unlock GPIO Port F

GPIO_PORTF_CR_R = 0x1F;           // allow changes to PF4-0

// Set LED pins as outputs

GPIOPinTypeGPIOOutput( GPIO_PORTF_BASE, RED_LED | GREEN_LED | BLUE_LED );

// Set PF0 as input

GPIOPinTypeGPIOInput( GPIO_PORTF_BASE, GPIO_PIN_0);

//register the interrupt handler for PF0

GPIOIntRegister(GPIO_PORTF_BASE, GPIOF0IntHandler);

//SW2 goes low when pressed

GPIOIntTypeSet(GPIO_PORTF_BASE, GPIO_PIN_0, GPIO_FALLING_EDGE);

//enable interrupts on PF0

GPIOIntEnable(GPIO_PORTF_BASE, GPIO_PIN_0);

}

void init_TIMER() {

/*

Configure the timer as periodic, by omission it's in count down mode.

It counts from the load value to 0 and then resets back to the load value.

*/

MAP_TimerConfigure(TIMER0_BASE, TIMER_CFG_PERIODIC);

ui32Period = SysCtlClockGet() / 2;  // Period of 0.5s 2Hz

MAP_TimerLoadSet(TIMER0_BASE, TIMER_A, ui32Period - 1);

MAP_IntEnable(INT_TIMER0A);

MAP_TimerIntEnable(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

}

void init_UART() {

// GPIO Setup

// Enable UART GPIO pins

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_UART0);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

// Configure GPIO pins for UART mode

MAP_GPIOPinConfigure(GPIO_PA0_U0RX);

MAP_GPIOPinConfigure(GPIO_PA1_U0TX);

MAP_GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

// Use internal 16MHz as clock

UARTClockSourceSet(UART0_BASE, UART_CLOCK_PIOSC);

// Initialize for I/O console

UARTStdioConfig(0, 115200, 16000000);

}

void Timer0AIntHandler(void) {

// Clear the interrupt

TimerIntClear(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

// Clear status flag before writing to ADC

ADCIntClear(ADC0_BASE, 1);

// Set ADC to trigger with software

ADCProcessorTrigger(ADC0_BASE, 1);

// Copy from ADC FIFO to buffer

ADCSequenceDataGet(ADC0_BASE, 1, ui32ADC0Value);

// Calculate average temperature

ui32TempAvg = (ui32ADC0Value[0] + ui32ADC0Value[1] + ui32ADC0Value[2] + ui32ADC0Value[3] + 2)/4;

// Convert to C (datasheet's equation)

ui32TempValueC = (1475 - ((2475 * ui32TempAvg)) / 4096)/10;

ui32TempValueF = ((ui32TempValueC * 9) + 160) / 5;

UARTprintf("F:%d\t", ui32TempValueF);

UARTprintf("C:%d\t\n", ui32TempValueC);

}

void GPIOF0IntHandler(void) { //interrupt handler for GPIO pin F0

// Clear interrupt flag on pin F0

MAP_GPIOIntClear(GPIO_PORTF_BASE, GPIO_PIN_0);

// Toggle all LEDs

int32_t status = GPIOPinRead(GPIO_PORTF_BASE, RED_LED|BLUE_LED|GREEN_LED);

MAP_GPIOPinWrite(GPIO_PORTF_BASE, RED_LED|BLUE_LED|GREEN_LED, 0xFFFFFFFF ^ status);

}

Task 2

/*  Created By: Rishawn Peppers Johnson

*  Date Created: 01 October 2020

*  Device: TivaC123GH6PM

*  CpE 403 Assignment 01 Task 02

*

*  Purpose: Using a user interface, execute the expected command. The possible commands are to

*          individually turn ON or OFF all three (R,G,B) LEDs, display the temperature in F or C,

*          and display the status of all the LEDs (ON or OFF). The PF4 button will toggle all

*          three (R,G,B) LEDs using an interrupt. The temperature value will be read and then

*          displayed in terminal through UART.

*

*  Inputs: Single character menu choice in terminal:

*      T       - Temperature in C

*      t       - Temperature in F

*      R/G/B   - Turn ON (R,G,B) LED (only one char at a time)

*      r/g/b   - Turn OFF (R,G,B) LED (only one char at a time)

*      S       - Display the status (ON or OFF) of LEDs

*

*  Outputs: Requested value or confirmation of command to terminal through UART.

*

*  */

#include <stdint.h>     // Variable definitions for C99 Standard

#include <stdbool.h>    // Boolean definitions for the C99 Standard

//#define TARGET_IS_BLIZZARD_RB1    // For rom.h, defines TivaC type

#include "inc/tm4c123gh6pm.h"   // def. for the interrupt and register assignments on the Tiva C Series device on the launchPad board

#include "inc/hw_memmap.h"      // Macros defining the memory map

#include "inc/hw_types.h"       // Common macros for TivaC

//#include "inc/hw_ints.h"      // Macros for TivaC interrupts

//#include "driverlib/rom.h"    // Macros for calling functions in ROM

#include "driverlib/rom_map.h"  // Macros for default calling functions in ROM otherwise in Flash

#include "utils/uartstdio.h"    // UART Console Driver Functions

#include "driverlib/interrupt.h"// Macros for NVIC Controller API of DriverLib

#include "driverlib/gpio.h"     // API GPIO ports

#include "driverlib/debug.h"    // Macros for debugging driverlib

#include "driverlib/sysctl.h"   // Driver for SysTick

#include "driverlib/systick.h"  // Driver for SysTick timer in NVIC

#include "driverlib/adc.h"      // Drivers for ADC

#include "driverlib/timer.h"    // Drivers for timers

#include "driverlib/uart.h"     // Macros for UART

#include "driverlib/pin_map.h"  // Mapping of peripherals and pins

// Library error routine

#ifdef DEBUG

void __error__(char *pcFilename, uint32_t ui32Line) {

}

#endif

// Pins

#define RED_LED   GPIO_PIN_1

#define BLUE_LED  GPIO_PIN_2

#define GREEN_LED GPIO_PIN_3

// Buffers

char cBuff[5];

// Variable0073

uint32_t ui32Period;

uint32_t ui32ADC0Value[4]; // Stores ADC value (size of ADC sequencer)

volatile uint32_t ui32TempAvg;

volatile uint32_t ui32TempValueC;

volatile uint32_t ui32TempValueF;

// Prototypes

// Custom Functions

void UARTprintLedStatus(uint32_t, uint8_t);

// Initialize Functions

void ConfigureUART(void);

void init_ADC();

void init_GPIO();

void init_TIMER();

// Interrupt Handlers

void Timer0AIntHandler(void);

void GPIOF0IntHandler(void);

void ADCseq0Handler(void);

// Program Entry

int main (void) {

// System clock to 40Mhz (PLL= 400Mhz / 10 = 40Mhz)

MAP_SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

// Enable Peripherals

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER0);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

MAP_SysCtlDelay(30u);

// Configure Peripherals

ConfigureUART();

UARTprintf("Terminal Active\n");

init_GPIO();

init_ADC();

init_TIMER();

MAP_IntMasterEnable(); // Enable Global Interrupts

MAP_TimerEnable(TIMER0_BASE, TIMER_A); // Start

while(1) {

ui32TempAvg = (ui32ADC0Value[0] + ui32ADC0Value[1] + ui32ADC0Value[2] + ui32ADC0Value[3] + 2)/4;

ui32TempValueC = (1475 - ((2475 * ui32TempAvg)) / 4096)/10;

ui32TempValueF = ((ui32TempValueC * 9) + 160) / 5;

// Read in from UART Buffer

UARTgets(cBuff, 2);

// Controls and UART Menu Display

if (cBuff[0] == 'R') { // Red LEDs

UARTprintf("Red LED ON\n");

GPIOPinWrite(GPIO_PORTF_BASE, RED_LED, RED_LED);

} else if( cBuff[0] == 'r') {

UARTprintf("Red LED OFF\n");

GPIOPinWrite(GPIO_PORTF_BASE, RED_LED, 0);

}

if (cBuff[0] == 'G') { // Green LEDs

UARTprintf("Green LED ON\n");

GPIOPinWrite(GPIO_PORTF_BASE, GREEN_LED, GREEN_LED);

} else if( cBuff[0] == 'g') {

UARTprintf("Green LED OFF\n");

GPIOPinWrite(GPIO_PORTF_BASE, GREEN_LED, 0);

}

if (cBuff[0] == 'B') { // Blue LEDs

UARTprintf("Blue LED ON\n");

GPIOPinWrite(GPIO_PORTF_BASE, BLUE_LED, BLUE_LED);

} else if( cBuff[0] == 'b') {

UARTprintf("Blue LED OFF\n");

GPIOPinWrite(GPIO_PORTF_BASE, BLUE_LED, 0);

}

if (cBuff[0] == 'T') // Temperature

UARTprintf("Temperature C: %d\n", ui32TempValueC);

else if( cBuff[0] == 't')

UARTprintf("Temperature F: %d\n", ui32TempValueF);

if (cBuff[0] == 'S') {   // LED Status

UARTprintf("LED Status:\n");

UARTprintf("Red LED:\t");

UARTprintLedStatus(GPIO_PORTF_BASE, RED_LED);

UARTprintf("Green LED:\t");

UARTprintLedStatus(GPIO_PORTF_BASE, GREEN_LED);

UARTprintf("Blue LED:\t");

UARTprintLedStatus(GPIO_PORTF_BASE, BLUE_LED);

}

}

}

void UARTprintLedStatus(uint32_t ui32Port, uint8_t ui8Pins) {

volatile int32_t status = GPIOPinRead(ui32Port, ui8Pins);

SysCtlDelay(1000);

if (status == 0)

UARTprintf("\tOFF\n");

else

UARTprintf("\tON\n");

}

void ConfigureUART(void) {

// GPIO setup for UART

// Enable GPIO Peripheral used by UART

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

// Enable UART0

SysCtlPeripheralEnable(SYSCTL_PERIPH_UART0);

// Configure GPIO pins for UART mode

GPIOPinConfigure(GPIO_PA0_U0RX);

GPIOPinConfigure(GPIO_PA1_U0TX);

GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

// Use internal 16MHz as clock

UARTClockSourceSet(UART0_BASE, UART_CLOCK_PIOSC);

// Initialize for I/O console

UARTStdioConfig(0, 115200, 16000000);

}

void init_ADC() {

// Disable interrupts on sequencer 0

ADCIntDisable( ADC0_BASE, 0 );

// Disable sample sequencer 0

ADCSequenceDisable( ADC0_BASE, 0 );

// Set interrupt function

ADCIntRegister( ADC0_BASE , 0, ADCseq0Handler );

// Using ADC0, sequencer 1, processor triggered, highest priority

ADCSequenceConfigure( ADC0_BASE, 0, ADC_TRIGGER_TIMER, 0 );

ADCSequenceStepConfigure(ADC0_BASE, 0, 0, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 0, 1, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 0, 2, ADC_CTL_TS);

// Configuration reading from internal temperature sensor as last in sequence.

// Causes interrupt when complete

ADCSequenceStepConfigure( ADC0_BASE, 0, 3, ADC_CTL_TS | ADC_CTL_END | ADC_CTL_IE );

// Re-enable sequencer 0

// Re-enable sequencer 0

ADCSequenceEnable(ADC0_BASE, 0 );

// Clear status flag before writing to ADC

ADCIntClear( ADC0_BASE, 0 );

// Enable DMA for sequencer 0

//ADCSequenceDMAEnable( ADC0_BASE, 0 );

// Re-enable interrupts for sequencer 0

ADCIntEnable( ADC0_BASE, 0 );

}

void init_GPIO() {

// Initialize GPIO pins

GPIO_PORTF_LOCK_R = 0x4C4F434B;   // unlock GPIO Port F

GPIO_PORTF_CR_R = 0x1F;           // allow changes to PF4-0

// Set LED pins as outputs

GPIOPinTypeGPIOOutput( GPIO_PORTF_BASE, RED_LED | GREEN_LED | BLUE_LED );

// Set PF0 as input

GPIOPinTypeGPIOInput( GPIO_PORTF_BASE, GPIO_PIN_0);

//register the interrupt handler for PF0

GPIOIntRegister(GPIO_PORTF_BASE, GPIOF0IntHandler);

//SW2 goes low when pressed

GPIOIntTypeSet(GPIO_PORTF_BASE, GPIO_PIN_0, GPIO_FALLING_EDGE);

//enable interrupts on PF0

GPIOIntEnable(GPIO_PORTF_BASE, GPIO_PIN_0);

}

void init_TIMER() {

TimerConfigure(TIMER0_BASE, TIMER_CFG_SPLIT_PAIR | TIMER_CFG_A_PERIODIC);

TimerLoadSet(TIMER0_BASE, TIMER_A, SysCtlClockGet()/16000 - 1);

// Enable ADC trigger output

TimerControlTrigger(TIMER0_BASE, TIMER_A, true);

// Timer stops when in debug mode

TimerControlStall(TIMER0_BASE, TIMER_A, true);

// Enable interrupt on Timer 0 A

//TimerIntEnable(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

TimerEnable(TIMER0_BASE, TIMER_A);

}

void Timer0AIntHandler(void) {

// Clear the interrupt

TimerIntClear(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

// Clear status flag before writing to ADC

ADCIntClear(ADC0_BASE, 1);

// Copy from ADC FIFO to buffer

ADCSequenceDataGet(ADC0_BASE, 0, ui32ADC0Value);

// Calculate average temperature

ui32TempAvg = (ui32ADC0Value[0] + ui32ADC0Value[1] + ui32ADC0Value[2] + ui32ADC0Value[3] + 2)/4;

// Convert to C (datasheet's equation)

ui32TempValueC = (1475 - ((2475 * ui32TempAvg)) / 4096)/10;

ui32TempValueF = ((ui32TempValueC * 9) + 160) / 5;

UARTprintf("F:%d\t", ui32TempValueF);

UARTprintf("C:%d\t\n", ui32TempValueC);

}

void GPIOF0IntHandler(void) { //interrupt handler for GPIO pin F0

// Clear interrupt flag on pin F0

MAP_GPIOIntClear(GPIO_PORTF_BASE, GPIO_PIN_0);

// Toggle all LEDs

int32_t status = GPIOPinRead(GPIO_PORTF_BASE, RED_LED|BLUE_LED|GREEN_LED);

MAP_GPIOPinWrite(GPIO_PORTF_BASE, RED_LED|BLUE_LED|GREEN_LED, 0xFFFFFFFF ^ status);

}

void ADCseq0Handler (void){

ADCSequenceDataGet(ADC0_BASE, 0, ui32ADC0Value);

ADCIntClear(ADC0_BASE, 0);

}

Task  3

/*  Created By: Rishawn Peppers Johnson

*  Date Created: 01 October 2020

*  Device: TivaC123GH6PM

*  CpE 403 Assignment 01 Task 03

*

*  Purpose: Using a user interface, execute the expected command. The possible commands are to

*          individually turn ON or OFF all three (R,G,B) LEDs, display the temperature in F or C,

*          and display the status of all the LEDs (ON or OFF). The PF4 button will toggle all

*          three (R,G,B) LEDs using an interrupt. The temperature value read from the ADC will

*          be stored in memory using uDMA. It will then be transfered to UART from memory to UART

*          using uDMA to be displayed in the terminal.

*

*  Inputs: Single character menu choice in terminal:

*      T       - Temperature in C

*      t       - Temperature in F

*      R/G/B   - Turn ON (R,G,B) LED (only one char at a time)

*      r/g/b   - Turn OFF (R,G,B) LED (only one char at a time)

*      S       - Display the status (ON or OFF) of LEDs

*

*  Outputs: Requested value or confirmation of command to terminal through UART.

*

*  */

#include <stdint.h>     // Variable definitions for C99 Standard

#include <stdbool.h>    // Boolean definitions for the C99 Standard

//#define TARGET_IS_BLIZZARD_RB1    // For rom.h, defines TivaC type

#include "inc/tm4c123gh6pm.h"   // def. for the interrupt and register assignments on the Tiva C Series device on the launchPad board

#include "inc/hw_memmap.h"      // Macros defining the memory map

#include "inc/hw_types.h"       // Common macros for TivaC

#include "inc/hw_adc.h"         // Macros for ADC hardware

#include "inc/hw_udma.h"        // Macros for uDMA registers

//#include "inc/hw_ints.h"      // Macros for TivaC interrupts

//#include "driverlib/rom.h"    // Macros for calling functions in ROM

#include "driverlib/rom_map.h"  // Macros for default calling functions in ROM otherwise in Flash

#include "utils/uartstdio.h"    // UART Console Driver Functions

#include "driverlib/interrupt.h"// Macros for NVIC Controller API of DriverLib

#include "driverlib/gpio.h"     // API GPIO ports

#include "driverlib/debug.h"    // Macros for debugging driverlib

#include "driverlib/sysctl.h"   // Driver for SysTick

#include "driverlib/systick.h"  // Driver for SysTick timer in NVIC

#include "driverlib/adc.h"      // Drivers for ADC

#include "driverlib/timer.h"    // Drivers for timers

#include "driverlib/uart.h"     // Macros for UART

#include "driverlib/udma.h"     // Drivers and macros for uDMA Controller

#include "driverlib/pin_map.h"  // Mapping of peripherals and pins

// Library error routine

#ifdef DEBUG

void __error__(char *pcFilename, uint32_t ui32Line) {

}

#endif

// uDMA Types

enum BUFFERSTATUS { EMPTY, FILLING, FULL };

// Pins

#define RED_LED   GPIO_PIN_1

#define BLUE_LED  GPIO_PIN_2

#define GREEN_LED GPIO_PIN_3

// Buffers

#define ADC_SAMPLE_BUF_SIZE     64

#define MEM_BUFFER_SIZE         1024

char cBuff[5];                                  // UART Input buffer

static uint32_t g_ui32SrcBuf[MEM_BUFFER_SIZE];

static uint32_t g_ui32DstBuf[MEM_BUFFER_SIZE];

static uint16_t ADC_Out1[ADC_SAMPLE_BUF_SIZE];

static uint16_t ADC_Out2[ADC_SAMPLE_BUF_SIZE];

static enum BUFFERSTATUS BufferStatus[2];

// uDMA control table aligned to 1024-byte boundary

#pragma DATA_ALIGN(ucControlTable, 1024)

uint8_t ucControlTable[1024];

// Variables

// Define transfer counter

static uint32_t g_ui32MemXferCount = 0;

// Define errors counters

static uint32_t g_ui32DMAErrCount = 0u;

static uint32_t g_ui32BadISR = 0u;

// Accumulator for system ticks

static uint32_t g_ui32SysTickCount;

// Prototypes

// Custom Functions

void UARTprintLedStatus(uint32_t, uint8_t);

// Initialize Functions

void ConfigureUART(void);

void init_DMA();

void init_GPIO();

void init_TIMER();

void init_ADC();

// Interrupt Handlers

void ADCseq0Handler(void);

void uDMAErrorHandler(void);

void GPIOF0IntHandler(void);

void SysTickIntHandler(void);

void Timer0AIntHandler(void);

// Program Entry

int main (void) {

// Variables

uint32_t i, average1, average2, samples_taken;

volatile uint32_t ui32TempAvg;

volatile uint32_t ui32TempValueC;

volatile uint32_t ui32TempValueF;

// System clock to 160Mhz (PLL= 400Mhz / 2.5 = 160Mhz)

SysCtlClockSet(SYSCTL_SYSDIV_2_5|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

// Enable Peripherals

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER0);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_UDMA);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOE);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

MAP_SysCtlDelay(30u);

// Enable System Clock

MAP_SysTickPeriodSet( SysCtlClockGet() / 100000u );

//SysTickIntRegister( SysTickIntHandler );

MAP_SysTickIntEnable();

MAP_SysTickEnable();

// Initialize

BufferStatus[0] = FILLING;

BufferStatus[1] = EMPTY;

samples_taken = 0u;

// Configure Peripherals

ConfigureUART();

UARTprintf("Terminal Active\n");

init_DMA();

init_GPIO();

init_ADC();

init_TIMER();

MAP_IntMasterEnable(); // Enable Global Interrupts

MAP_TimerEnable(TIMER0_BASE, TIMER_A); // Start

while(1) {

// Read into Buffer1 or Buffer2 if the other is full

if (BufferStatus[0u] == FULL) {

average1 = 0u; // Reset average to accumulate

// Sum and clear values from buffer

for(i = 0u; i < ADC_SAMPLE_BUF_SIZE; i++) {

average1 += ADC_Out1[i];

ADC_Out1[i] = 0u;

}

BufferStatus[0u] = EMPTY; // Clear buffer

// Mark and enable to be transfered into ADC_Out1

uDMAChannelTransferSet( UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT, UDMA_MODE_PINGPONG,

(void *)(ADC0_BASE + ADC_O_SSFIFO0), &ADC_Out1, ADC_SAMPLE_BUF_SIZE );

uDMAChannelEnable( UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT );

// Update sample count and average

samples_taken += ADC_SAMPLE_BUF_SIZE;

average1 = (average1 + ( ADC_SAMPLE_BUF_SIZE / 2u)) / ADC_SAMPLE_BUF_SIZE;

}

if (BufferStatus[1u] == FULL) {

average2 = 0u; // Reset average to accumulate

// Sum and clear values from buffer

for(i = 0u; i < ADC_SAMPLE_BUF_SIZE; i++) {

average2 += ADC_Out2[i];

ADC_Out2[i] = 0u;

}

BufferStatus[1u] = EMPTY;  // Clear buffer

// Mark and enable to be transfered into ADC_Out2

uDMAChannelTransferSet( UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT, UDMA_MODE_PINGPONG,

(void *)(ADC0_BASE + ADC_O_SSFIFO0), &ADC_Out2, ADC_SAMPLE_BUF_SIZE );

uDMAChannelEnable( UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT );

// Update sample count and average

samples_taken += ADC_SAMPLE_BUF_SIZE;

average2 = (average2 + ( ADC_SAMPLE_BUF_SIZE / 2u)) / ADC_SAMPLE_BUF_SIZE;

// UARTprintf("\t%d\t\t%d\t\t%d\r", ui32TempValueC, ui32TempValueF, samples_taken);

}

// Calculate Temperature in C and F

ui32TempAvg = (average1 + average2) / 2;

ui32TempValueC = (1475 - ((2475 * ui32TempAvg)) / 4096)/10;

ui32TempValueF = ((ui32TempValueC * 9) + 160) / 5;

// Read in from UART Buffer

UARTgets(cBuff, 2);

// Controls and UART Menu Display

if (cBuff[0] == 'R') { // Red LEDs

UARTprintf("Red LED ON\n");

GPIOPinWrite(GPIO_PORTF_BASE, RED_LED, RED_LED);

} else if( cBuff[0] == 'r') {

UARTprintf("Red LED OFF\n");

GPIOPinWrite(GPIO_PORTF_BASE, RED_LED, 0);

}

if (cBuff[0] == 'G') { // Green LEDs

UARTprintf("Green LED ON\n");

GPIOPinWrite(GPIO_PORTF_BASE, GREEN_LED, GREEN_LED);

} else if( cBuff[0] == 'g') {

UARTprintf("Green LED OFF\n");

GPIOPinWrite(GPIO_PORTF_BASE, GREEN_LED, 0);

}

if (cBuff[0] == 'B') { // Blue LEDs

UARTprintf("Blue LED ON\n");

GPIOPinWrite(GPIO_PORTF_BASE, BLUE_LED, BLUE_LED);

} else if( cBuff[0] == 'b') {

UARTprintf("Blue LED OFF\n");

GPIOPinWrite(GPIO_PORTF_BASE, BLUE_LED, 0);

}

if (cBuff[0] == 'T') // Temperature

UARTprintf("Temperature C: %d\n", ui32TempValueC);

else if( cBuff[0] == 't')

UARTprintf("Temperature F: %d\n", ui32TempValueF);

if (cBuff[0] == 'S') {   // LED Staus

UARTprintf("LED Status:\n");

SysCtlDelay(1000);

UARTprintf("Red LED:\t");

UARTprintLedStatus(GPIO_PORTF_BASE, RED_LED);

UARTprintf("Green LED:\t");

UARTprintLedStatus(GPIO_PORTF_BASE, GREEN_LED);

UARTprintf("Blue LED:\t");

UARTprintLedStatus(GPIO_PORTF_BASE, BLUE_LED);

}

}

}

void UARTprintLedStatus(uint32_t ui32Port, uint8_t ui8Pins) {

// Display ON or OFF to UART

volatile int32_t status = GPIOPinRead(ui32Port, ui8Pins);

SysCtlDelay(1000);

if (status == 0)

UARTprintf("\tOFF\n");

else

UARTprintf("\tON\n");

}

void ConfigureUART(void) {

// GPIO setup for UART

// Enable UART GPIO pins

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_UART0);

// Configure GPIO pins for UART mode

MAP_GPIOPinConfigure(GPIO_PA0_U0RX);

MAP_GPIOPinConfigure(GPIO_PA1_U0TX);

MAP_GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

// Use internal 16MHz as clock

UARTClockSourceSet(UART0_BASE, UART_CLOCK_PIOSC);

// Initialize for I/O console

UARTStdioConfig(0, 115200, 16000000);

}

void init_DMA() {

// Initialize uDMA on ADC CH0 in Ping-Pong Mode

uDMAEnable();

uDMAControlBaseSet(ucControlTable);

uDMAChannelAttributeDisable( UDMA_CHANNEL_ADC0, UDMA_ATTR_ALTSELECT |

UDMA_ATTR_HIGH_PRIORITY| UDMA_ATTR_REQMASK);

uDMAChannelAttributeEnable( UDMA_CHANNEL_ADC0, UDMA_ATTR_USEBURST );

uDMAChannelControlSet( UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT,

UDMA_SIZE_16 |UDMA_SRC_INC_NONE | UDMA_DST_INC_16 | UDMA_ARB_1);

uDMAChannelControlSet( UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT,

UDMA_SIZE_16 | UDMA_SRC_INC_NONE | UDMA_DST_INC_16 | UDMA_ARB_1);

uDMAChannelTransferSet( UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT, UDMA_MODE_PINGPONG,

(void *)(ADC0_BASE + ADC_O_SSFIFO0), &ADC_Out1, ADC_SAMPLE_BUF_SIZE);

uDMAChannelTransferSet( UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT, UDMA_MODE_PINGPONG,

(void *)(ADC0_BASE + ADC_O_SSFIFO0), &ADC_Out2, ADC_SAMPLE_BUF_SIZE);

uDMAChannelEnable(UDMA_CHANNEL_ADC0);

}

void init_GPIO() {

// Initialize GPIO pins

GPIO_PORTF_LOCK_R = 0x4C4F434B;   // unlock GPIO Port F

GPIO_PORTF_CR_R = 0x1F;           // allow changes to PF4-0

// Set LED pins as outputs

GPIOPinTypeGPIOOutput( GPIO_PORTF_BASE, RED_LED | GREEN_LED | BLUE_LED );

// Set PF0 as input

GPIOPinTypeGPIOInput( GPIO_PORTF_BASE, GPIO_PIN_0);

//register the interrupt handler for PF0

GPIOIntRegister(GPIO_PORTF_BASE, GPIOF0IntHandler);

//SW2 goes low when pressed

GPIOIntTypeSet(GPIO_PORTF_BASE, GPIO_PIN_0, GPIO_FALLING_EDGE);

//enable interrupts on PF0

GPIOIntEnable(GPIO_PORTF_BASE, GPIO_PIN_0);

}

void init_TIMER() {

// Initialize Timer 0

MAP_TimerConfigure(TIMER0_BASE, TIMER_CFG_SPLIT_PAIR | TIMER_CFG_A_PERIODIC);

MAP_TimerLoadSet(TIMER0_BASE, TIMER_A, SysCtlClockGet()/16000 - 1);

// Enable ADC trigger output

MAP_TimerControlTrigger(TIMER0_BASE, TIMER_A, true);

// Timer stops when in debug mode

MAP_TimerControlStall(TIMER0_BASE, TIMER_A, true);

// Enable interrupt on Timer 0 A

TimerIntRegister(TIMER0_BASE, TIMER_A, Timer0AIntHandler);

MAP_TimerIntEnable(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

}

void init_ADC() {

// ADC CH0 Timer Triggered for uDMA

// Set GPIO PIN to read in ADC

GPIOPinTypeADC(GPIO_PORTE_BASE, GPIO_PIN_2);

SysCtlDelay(80);

// Set ADC clock as PIOSC / 1 at half rate (16MHz/1) / 2 = 8 MHz

ADCClockConfigSet( ADC0_BASE, ADC_CLOCK_SRC_PIOSC | ADC_CLOCK_RATE_HALF, 1 );

// Wait for configuration to finish

SysCtlDelay(10);

// Disable interrupts on ADC0

IntDisable( INT_ADC0SS0 );

// Disable interrupts on sequencer 0

ADCIntDisable( ADC0_BASE, 0 );

// Disable sample sequencer 0

ADCSequenceDisable( ADC0_BASE, 0 );

// Set interrupt function

//ADCIntRegister( ADC0_BASE , 0, ADCseq0Handler );

// Using ADC0, sequencer 1, processor triggered, highest priority

ADCSequenceConfigure( ADC0_BASE, 0, ADC_TRIGGER_TIMER, 0 );

// Configuration reading from internal temperature sensor as last in sequence.

// Causes interrupt when complete

ADCSequenceStepConfigure( ADC0_BASE, 0, 0, ADC_CTL_TS | ADC_CTL_END | ADC_CTL_IE );

// Re-enable sequencer 0

// Re-enable sequencer 0

ADCSequenceEnable(ADC0_BASE, 0 );

// Clear status flag before writing to ADC

ADCIntClear( ADC0_BASE, 0 );

// Enable DMA for sequencer 0

ADCSequenceDMAEnable( ADC0_BASE, 0 );

// Re-enable interrupts for sequencer 0

ADCIntEnable( ADC0_BASE, 0 );

// Re-enable interrupts on ADC 0

IntEnable( INT_ADC0SS0 );

}

void ADCseq0Handler (void){

// ADCSequenceDataGet(ADC0_BASE, 0, ui32ADC0Value);

//    UARTprintf("Seq Interrupt");

ADCIntClear(ADC0_BASE, 0);

if((uDMAChannelModeGet(UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT) == UDMA_MODE_STOP)

&& (BufferStatus[0] == FILLING))

{

BufferStatus[0] = FULL;

BufferStatus[1] = FILLING;

} else if ((uDMAChannelModeGet(UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT) == UDMA_MODE_STOP)

&& (BufferStatus[1] == FILLING))

{

BufferStatus[0] = FILLING;

BufferStatus[1] = FULL;

}

}

// uDMA error handler

void uDMAErrorHandler(void) {

uint32_t ui32Status;

ui32Status = uDMAErrorStatusGet();

if(ui32Status)

{

uDMAErrorStatusClear();

g_ui32DMAErrCount++;

}

}

void GPIOF0IntHandler(void) {

// Clear interrupt flag on pin F0

GPIOIntClear(GPIO_PORTF_BASE, GPIO_PIN_0);

// Toggle all LEDs

int32_t status = GPIOPinRead(GPIO_PORTF_BASE, RED_LED|BLUE_LED|GREEN_LED);

GPIOPinWrite(GPIO_PORTF_BASE, RED_LED|BLUE_LED|GREEN_LED, 0xFFFFFFFF ^ status);

}

void SysTickIntHandler(void) {

g_ui32SysTickCount++;

//    UARTprintf("Sys Tick ");

}

// For debugging purposes

void Timer0AIntHandler(void) {

// Clear the interrupt

TimerIntClear(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

//    UARTprintf("Timer Int ");

}

- Block diagram and/or Schematics showing the components, pins used, and interface.

- Screenshots of the IDE, physical setup, debugging process - Provide screenshot of successful compilation, screenshots of registers, variables, graphs, etc.

- Declaration

I understand the Student Academic Misconduct Policy - http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Rishawn Peppers Johnson